APPENDIX D
HISTORICAL DATA AND TREND EVALUATION

Data Use Summary and Trend Plate Plotting Conventions

2025 Annual Monitoring Report
SS045
Former England Air Force Base
Alexandria, Louisiana

Working Copy – For Internal Development and Air Force Review

Table D-0. Appendix D Document Control

| Prepared for | 2025 Annual Monitoring Report, SS045 |
| --- | --- |
| Purpose | Define data-use conventions, trend-plate content, plotting assumptions, and QA/QC expectations for Appendix D. |
| Primary use | Support the main-report remedy-performance interpretation and 2026 RA-O monitoring recommendations. |
| Status | Working copy; update after final trend plates, final table values, and AF review comments are incorporated. |
| Save location | 04_Trend_Plates |

# Acronyms and Abbreviations

| AMR | Annual Monitoring Report |
| --- | --- |
| BCT-CSL | BRAC Cleanup Team-Concurred Screening Level |
| COC | contaminant of concern |
| DCE | dichloroethene |
| DO | dissolved oxygen |
| DUVR | Data Verification and Usability Report |
| LUC | land use control |
| MNA | monitored natural attenuation |
| ORP | oxidation-reduction potential |
| QAPP | Quality Assurance Project Plan |
| RA-O | Remedial Action-Operations |
| TCE | trichloroethene |
| TOC | total organic carbon |
| VC | vinyl chloride |
| VOC | volatile organic compound |
| CMIWP | Corrective Measures Implementation Work Plan |
| OPS | operating properly and successfully |
| QA/QC | quality assurance/quality control |
| SS045 | Spill Site 45 / SWMU 332 |

# 1. Purpose and Use of Appendix D

This appendix supports the 2025 AMR by providing the historical-data context and priority-well trend plates used to evaluate MNA performance at SS045. The appendix is intended to function as the technical proof behind the concise main-report narrative; it should not repeat the full site history, regulatory history, or field documentation. The main report will summarize the remedy-performance conclusion, while Appendix D will provide the well-specific trend evidence, plotting conventions, and historical-data support used to reach that conclusion.

The appendix is organized around priority wells rather than every monitoring location. Priority wells include current-year BCT-CSL exceedance wells, wells with historical trend significance, deep-zone performance wells, and selected wells needed to support 2026 RA-O monitoring recommendations. This approach is consistent with EPA guidance for MNA performance monitoring and long-term monitoring optimization, which emphasizes site-specific evidence, contaminant trends, geochemical conditions, and monitoring networks focused on remedy-performance objectives.

# 2. Evaluation Framework

SS045 is evaluated as a single groundwater plume with site areas used for remedy-performance interpretation. North Apron, Tower, Central/BH-01, and Le Tig Bayou are reporting areas within the SS045 plume and should not be presented as separate regulatory plumes unless future concurrence documentation establishes that terminology.

The 2025 AMR evaluates current groundwater conditions using individual COC concentrations compared to BCT-CSLs, concentration trends, MNA indicators, groundwater-flow conditions, data usability, and LUC status. Historical interim goals are not used as current annual performance criteria.

Trend plates are interpretive support tools. They do not replace the validated analytical tables, final exceedance summary, DUVR, or main-report recommendations. Where a trend plate and a final report table differ, the final validated analytical table and the main-report table should control.

# 3. Data Sources

Table D-1. Data Sources Used for Appendix D

| Data source | Use in Appendix D | Primary report link |
| --- | --- | --- |
| 2025 validated groundwater analytical results | COC concentrations, duplicate results, reporting limits, and qualifiers used for exceedance status and trend-plate updates. | Appendix B and Table 3-1 |
| 2025 DUVR | Validation basis, data usability, completeness, and qualifier limitations. | Appendix C and Table 3-2 |
| Historical groundwater data summaries | Historical time-series data used for trend plots and prior-event comparisons. | Appendix D electronic workbook |
| 2025 field parameter and MNA data | DO, ORP, methane, ethene, ethane, TOC, chloride, sulfate, nitrate/nitrite, and related indicators. | Table 3-3 and Appendix A/B/C |
| 2025 water-level and well inspection data | Groundwater elevation, inspection status, well maintenance observations, and network exceptions. | Appendix A and Figures 3-1/3-2 |
| Precipitation data | Context for water-level response and seasonal interpretation; not used alone to infer contaminant behavior. | Trend plate input workbook |
| Work Plan, CMIWP, and OPS Report | RA-O scope, remedy framework, LUC status, and historical precedent for trend-analysis support appendices. | References |

# 4. Priority Well Trend Plate Set

Appendix D will include 11-by-17-inch landscape trend plates for the priority wells listed in Table D-2. The plate set is intentionally limited to wells needed to support the 2025 remedy-performance interpretation and 2026 RA-O recommendations. Additional wells may be added only if AF review, final analytical QA/QC, or trend-plate development identifies a specific interpretive need.

Table D-2. Priority Well Trend Plate Index

| Plate | Well | Site area | Zone | COCs to plot | Interpretation focus |
| --- | --- | --- | --- | --- | --- |
| D-1 | BH01MW02 | Central/BH-01 | Shallow | Benzene, VC | Central/BH-01 exceedance. |
| D-2 | Well #8 | Le Tig Bayou | Intermediate | VC | Localized VC exceedance. |
| D-3 | Well #25 | Le Tig Bayou | Intermediate | VC | Current-year VC exceedance. |
| D-4 | A39L014DP | North Apron | Intermediate | TCE | Low-level TCE exceedance. |
| D-5 | A39MW095 | North Apron | Intermediate | VC | Low-level VC exceedance. |
| D-6 | SS45L001MW | North Apron | Intermediate | VC | VC exceedance and sample-ID note. |
| D-7 | Well #11 | North Apron | Intermediate | VC | Primary VC performance location. |
| D-8 | Well #13 | North Apron | Intermediate | VC | Primary VC performance location. |
| D-9 | Well #15 | North Apron | Intermediate | 1,1-DCE, TCE, VC | Multiple COC exceedances. |
| D-10 | Well #17 | North Apron | Intermediate | VC | Persistent VC exceedance. |
| D-11 | A39L028DP | North Apron | Deep | VC | Deep-zone VC exceedance. |
| D-12 | A39MW094 | North Apron | Deep | cis-1,2-DCE, VC | Primary deep-zone issue. |
| D-13 | SS45L002MW | North Apron | Deep | VC | Deep-zone VC exceedance. |
| D-14 | Well #12 | North Apron | Deep | VC | Deep-zone VC exceedance. |
| D-15 | Well #14 | North Apron | Deep | VC | Low-level deep-zone VC exceedance. |
| D-16 | A39L036DPR2 | Tower | Intermediate | TCE | Current-year TCE exceedance. |

# 5. Trend Plate Layout and Plotting Conventions

Table D-3. Required Trend Plate Components

| Plate component | Requirement |
| --- | --- |
| Header block | Well ID, site area, groundwater zone, monitoring category, screen interval where available, 2025 exceedance status, and recommended 2026 RA-O action. |
| VOC concentration plot | Primary COC time series on a log-scale y-axis. Plot only COCs needed to understand the well-specific story. Include BCT-CSL reference lines and label the 2025 result. |
| Hydrograph / precipitation context | Groundwater elevation and/or precipitation panel used to support hydrologic context. Precipitation should not be treated as proof of contaminant causation. |
| Recent-event table | Last three sampling events, detected COCs, BCT-CSL comparison, and key qualifiers. |
| Trend / interpretation box | Long-term trend, recent direction, MNA indicator context, and 2026 recommended action in concise language. |
| Notes block | Nondetect plotting convention, duplicate handling, qualifiers, data source, and units/acronyms for standalone use. |

## 5.1 Plotting Rules

Use a log-scale y-axis for VOC concentration plots unless all values are consistently low and a linear scale provides clearer interpretation. If a linear scale is used, state that on the plate.

Plot detected results at the reported concentration. Estimated detects (J-qualified results) should be plotted as detections and identified in the notes.

Plot nondetect results at the reporting limit or method detection limit used by the historical data table, using an open symbol. The plate must state the nondetect convention.

Do not connect a trend line through long gaps without showing the actual sampling dates. Use the date axis rather than equal spacing between events.

Use BCT-CSL reference lines for COCs with screening levels.

Where parent and duplicate results are available, use the parent sample as the primary plotted result and show the duplicate in the recent-event table or as a paired marker if needed for interpretation.

Do not use precipitation, groundwater elevation, or ORP/DO alone as evidence of MNA performance.

# 6. Trend Interpretation Conventions

Trend plates should distinguish long-term statistical trend, recent concentration behavior, and current-year exceedance status. These are related but not interchangeable. A well can show a long-term increasing trend while also showing a recent decrease from a historical peak; the plate and main-report text must identify that distinction explicitly.

Table D-4. Trend Interpretation Categories

| Category | Definition | Use limitation |
| --- | --- | --- |
| Long-term trend | Mann-Kendall or equivalent historical trend result where the dataset supports a valid statistical assessment. | Use only after final trend calculations are confirmed. |
| Recent direction | Comparison of recent events, generally the last three sampling events or the period most relevant to current remedy performance. | Use to explain current-year behavior without overstating statistical significance. |
| Current exceedance status | 2025 COC result compared to applicable BCT-CSL. | Controls whether the well is included in the main exceedance table and priority plate set. |
| MNA interpretation | Combined interpretation of parent/daughter product behavior, reducing conditions, dissolved gases, and geochemical indicators. | Avoid conclusions based on a single indicator. |
| 2026 action | Recommended monitoring category, follow-up, or optimization action. | Must be tied to monitoring objectives or a 2025 finding. |

# 7. Data Qualifiers and Use Limitations

Validated data should be used as reported. Qualifiers that affect interpretation should be carried into the plate notes and cross-referenced to the DUVR. The main report should summarize only those limitations that affect remedy-performance interpretation.

Table D-5. Qualifier and Data-Use Conventions

| Qualifier / issue | Meaning | Appendix D treatment |
| --- | --- | --- |
| U | Analyte not detected at or above the associated reporting limit. | Plot as nondetect at the reporting limit using open symbol; do not treat as zero. |
| J | Estimated detected concentration. | Plot as detection and note estimated status. |
| UJ | Estimated nondetect/reporting limit. | Plot as nondetect at estimated reporting limit; note limitation if used in interpretation. |
| R | Rejected result. | Do not use for trend or exceedance interpretation unless a qualified professional documents a specific limited use. |
| Field duplicate | Independent field QC sample. | Use to evaluate reproducibility; do not duplicate-count exceedances. |
| A39L092DP limitation | 2025 VOC nondetects were qualified UJ due to bottle headspace/bubble, and iron/chloride/TOC were qualified J due to duplicate criterion exceedance. | Use cautiously for MNA interpretation; footnote any plate or table that relies on A39L092DP. |

# 8. Appendix D QA/QC Checklist

[ ] Plate well ID matches Table 3-1, Table 4-1, Figure 4-1, and the historical data workbook.

[ ] Plate site area and groundwater zone match the monitoring-scope matrix and GIS input table.

[ ] COCs plotted are limited to those needed for the well-specific remedy-performance story.

[ ] BCT-CSL lines and values match Table 2-1.

[ ] 2025 result labels match the final analytical summary and DUVR qualifiers.

[ ] Nondetect plotting convention is stated on each plate.

[ ] Parent/duplicate sample handling is documented and duplicate results are not double-counted.

[ ] Long-term trend and recent direction are not conflated.

[ ] MNA interpretation is supported by VOC trends plus geochemical/dissolved-gas evidence where available.

[ ] 2026 action statement matches Table 5-1 and does not introduce unsupported recommendations.

# 9. Limitations

Appendix D is a trend-support appendix and is not intended to re-establish the remedy, recreate the historical site investigation record, or replace the final validated analytical tables.

Trend plates are intended to support annual remedy-performance evaluation and monitoring optimization. They are not a substitute for a formal statistical trend report unless the final AMR explicitly adopts them for that purpose.

Historical data may include different analytical methods, reporting limits, sample timing, and validation practices. These differences should be considered when interpreting older data.

Precipitation and groundwater-elevation panels provide context but do not independently establish cause-and-effect relationships for VOC concentration changes.

The final Appendix D package should be updated after final analytical table QA/QC, trend-plate production, AF review, and any required backcheck.

# 10. References

FPM Remediations, Inc. (FPM). 2023. Final Amended Work Plan for Remedial Action-Operation at SS045 (SWMU 332) and for Long-Term Management at LF015 (SWMU 41), former England Air Force Base, Alexandria, Louisiana. June.

FPM. 2026. Final 2026 Basewide Corrective Measures Implementation Work Plan for Land Use Controls, Former England Air Force Base, Alexandria, Louisiana. April.

Shaw Environmental & Infrastructure, Inc. (Shaw). 2010. Final Operating Properly and Successfully Report, Site SS-45, Former England Air Force Base, Louisiana. October.

U.S. Environmental Protection Agency (USEPA). 1998. Technical Protocol for Evaluating Natural Attenuation of Chlorinated Solvents in Ground Water. EPA/600/R-98/128. September.

USEPA. 1999. Use of Monitored Natural Attenuation at Superfund, RCRA Corrective Action, and Underground Storage Tank Sites. OSWER Directive 9200.4-17P. April 21.

USEPA and U.S. Army Corps of Engineers. 2005. Roadmap to Long-Term Monitoring Optimization. EPA 542-R-05-003. May.

2025 SS045 Data Verification and Usability Report, analytical reports, field documentation, and historical data summary workbooks prepared for the 2025 AMR.